Assignment -4

| Assignment Date | 02 NOVEMBER 2022 |
| --- | --- |
| Student Name | DIVYA L |
| Student Roll Number | 727819TUEC031 |
| Maximum Marks | 2 Marks |

Question-1:

## Write code and connections in wowki for ultrasonic sensor.

Whenever distance is less than 100 cms send “alert” to IBM cloud and display in device recent events.

# Solution:

WOWKI LINK:    https://wokwi.com/projects/346235465961046612

#include <WiFi.h>

#include <PubSubClient.h>

#define TRIGGER 2

#define ECHO 15

#define sound_speed 0.034

int distance;

void callback(char* subscribetopic, byte* payload, unsigned int payloadLength);

//-------credentials of IBM Accounts------

#define ORG "wp72r7"

#define DEVICE_TYPE "iot-device-1"

#define DEVICE_ID "123456789"

#define TOKEN "987654321"

String data3;

//-------- Customise the above values --------

char server[] = ORG ".messaging.internetofthings.ibmcloud.com"; char publishTopic[] = "iot-2/evt/Data/fmt/json"; char subscribetopic[] = "iot-2/cmd/test/fmt/String"; char authMethod[] = "use-token-auth"; char token[] = TOKEN;  char clientId[] = "d:" ORG ":" DEVICE_TYPE ":" DEVICE_ID;

//----------------------------------------- WiFiClient wifiClient;

PubSubClient client(server, 1883, callback ,wifiClient); void setup()

{

Serial.begin(115200);   pinMode(TRIGGER, OUTPUT);    pinMode(ECHO, INPUT);

delay(10);   Serial.println();

wificonnect();

mqttconnect();

}

void loop()

{

digitalWrite(TRIGGER, HIGH);

delayMicroseconds(10);   digitalWrite(TRIGGER,

LOW);

int duration=pulseIn(ECHO,HIGH);   distance=(duration*sound_speed)/2;

Serial.print("Distance:");

Serial.print(distance);

Serial.println("cms");   if(distance<100){     PublishData(distance);

}

delay(1000);   if (!client.loop())

{

mqttconnect();

}

}

/.....................................retrieving to Cloud.............................../

void PublishData(int d) {    mqttconnect();

String payload = "{\"message\":alert}";

Serial.print("Sending payload: ");

Serial.println(payload);

if (client.publish(publishTopic, (char*) payload.c_str())) {

Serial.println("Publish ok");

} else {

Serial.println("Publish failed");

}

}

void mqttconnect() {   if (!client.connected())

{

Serial.print("Reconnecting client to ");      Serial.println(server);       while (!!!client.connect(clientId, authMethod, token)) {        Serial.print(".");         delay(500);

}

initManagedDevice();

Serial.println();

}

}

void wificonnect()

{

Serial.println();

Serial.print("Connecting to ");

WiFi.begin("Wokwi-GUEST", "", 6);   while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");   }

Serial.println("");

Serial.println("WiFi connected");

Serial.println("IP address: ");

Serial.println(WiFi.localIP());

}

void initManagedDevice() {

if (client.subscribe(subscribetopic)) {     Serial.println((subscribetopic));

Serial.println("subscribe to cmd OK");

} else {

Serial.println("subscribe to cmd FAILED");

}

}

void callback(char* subscribetopic, byte* payload, unsigned int payloadLength)

{

Serial.print("callback invoked for topic: ");

Serial.println(subscribetopic);   for (int i =

0; i < payloadLength; i++) {     data3 +=

(char)payload[i];

}

Serial.println("data: "+ data3);

data3="";

}

CIRCUIT DIAGRAM:

IBM CLOUD RECENT EVENTS: